# OPERATION CODE BLUE

Wazuh VM Setup, Current Detection Rules, and Analyst Queue

Purpose: Use this guide to build the Wazuh portion of the hospital capstone exactly as it is currently configured: two CDB lists, five custom rules, Windows telemetry, testing, and the dashboard SOC queue.

Safety: Fictional training environment only. Run Nmap and SMB commands only against the isolated 192.168.56.0/24 VirtualBox lab.

# How to Use This Guide

| Label | Where to run it | Purpose |
| --- | --- | --- |
| REAL COMPUTER | Physical Windows computer | Open VirtualBox, dashboard, and PowerShell SSH. |
| WINDOWS VM | WIN-PATIENT-SERVER | Install the agent and enable Windows auditing. |
| WAZUH SSH | SSH session to 192.168.56.103 | Create lists and rules; manage the Wazuh manager. |
| KALI VM | Attacker simulation VM | Run the portal, Nmap, smbclient, and safe .locked rename. |

## Lab Addresses

| System | Address | Role |
| --- | --- | --- |
| Kali | 192.168.56.101 | Attacker simulation and portal host |
| Windows | 192.168.56.102 | Hospital workstation and PatientRecords server |
| Wazuh | 192.168.56.103 | SIEM manager and dashboard |
| Portal | identity.blueridgeregional.net:5000 | Fictional identity-verification page |

Address note: The CDB list values and lab commands below use these addresses. If VirtualBox DHCP assigns different addresses, update the lists and commands before testing. Rule 100102 intentionally keeps the protected Windows asset address fixed.

# Part A: Build and Start the Wazuh VM

## A1. Import and network the appliance

Open Oracle VirtualBox and select File > Import Appliance.

Choose the Wazuh OVA used by the group and complete the import.

Attach at least one adapter to the same Host-Only network used by Kali and Windows.

Optionally keep a NAT adapter for updates, while the demonstration traffic remains on Host-Only.

Start the Wazuh VM and log in with the lab credentials.

ip -br address

Confirm that the Host-Only address is 192.168.56.103 or record the address assigned in your copy of the lab.

## A2. Open the dashboard and SSH

Dashboard: https://192.168.56.103

PowerShell SSH: ssh wazuh-user@192.168.56.103

Lab certificate: A browser certificate warning is expected because the isolated lab uses a self-signed certificate.

# Part B: Install the Wazuh Agent on Windows

## B1. Verify connectivity

Test-NetConnection 192.168.56.103 -Port 1514 Test-NetConnection 192.168.56.103 -Port 1515

Both commands should show TcpTestSucceeded : True.

## B2. Deploy the agent

In the Wazuh dashboard, open Endpoints and choose Deploy new agent.

Select Windows, 64-bit architecture, manager 192.168.56.103, and agent name WIN-PATIENT-SERVER.

Copy the generated PowerShell installation command and run it inside Administrator PowerShell on Windows.

Start the Wazuh service and wait for the agent to become Active.

Start-Service wazuhsvc Get-Service wazuhsvc

# Part C: Enable Required Windows Telemetry

## C1. Enable audit policy

auditpol /set /subcategory:"Logon" /success:enable /failure:enable auditpol /set /subcategory:"File Share" /success:enable /failure:enable auditpol /set /subcategory:"Detailed File Share" /success:enable /failure:enable auditpol /set /subcategory:"File System" /success:enable /failure:enable auditpol /set /subcategory:"Process Creation" /success:enable auditpol /set /subcategory:"Filtering Platform Connection" /success:enable /failure:disable auditpol /set /subcategory:"Filtering Platform Packet Drop" /failure:enable

## C2. Verify firewall-related audit settings

auditpol /get /subcategory:"Filtering Platform Connection" auditpol /get /subcategory:"Filtering Platform Packet Drop"

## C3. Add folder auditing to PatientRecords

Open C:\HospitalLab\PatientRecords in File Explorer.

Right-click PatientRecords and open Properties > Security > Advanced > Auditing.

Add Everyone as the principal for this isolated lab.

Set Type to Success and Applies to This folder, subfolders and files.

Audit read/list, write/create, append, delete, and the operations used in the demonstration.

Why this matters: Audit policy alone is not enough for Event 4663. The PatientRecords folder also needs a System Access Control List auditing entry.

# Part D: Configure the Windows Agent to Forward Events

Important: Keep exactly one <location>Security</location> source. Do not create a duplicate Security localfile block.

## D1. Back up ossec.conf and remove exclusions

$path = "C:\Program Files (x86)\ossec-agent\ossec.conf" $backup = "$path.before-operation-code-blue.bak" Copy-Item $path $backup -Force $text = [System.IO.File]::ReadAllText($path) $eventIds = 4663, 5145, 5156, 5152 foreach ($eventId in $eventIds) { $text = [regex]::Replace($text, "(?i)\s+and\s+EventID\s*!=\s*$eventId\b", "") $text = [regex]::Replace($text, "(?i)\bEventID\s*!=\s*$eventId\s+and\s+", "") } $utf8 = New-Object System.Text.UTF8Encoding($false) [System.IO.File]::WriteAllText($path, $text, $utf8)

## D2. Verify and restart the agent

Select-String -Path "C:\Program Files (x86)\ossec-agent\ossec.conf" -Pattern "<location>Security</location>" Select-String -Path "C:\Program Files (x86)\ossec-agent\ossec.conf" -Pattern "EventID != 4663¦EventID != 5145¦EventID != 5156¦EventID != 5152" Stop-Service wazuhsvc Start-Sleep -Seconds 8 Start-Service wazuhsvc Get-Service wazuhsvc

The first Select-String should return exactly one Security source. The second should return no output. The Wazuh service should show Running.

# Part E: Create and Register the CDB Lists

## E1. Create the project IOC list

sudo tee /var/ossec/etc/lists/ocblue_suspicious_destinations > /dev/null <<'EOF' 192.168.56.101:simulated-phishing-portal EOF sudo chown wazuh:wazuh /var/ossec/etc/lists/ocblue_suspicious_destinations sudo chmod 660 /var/ossec/etc/lists/ocblue_suspicious_destinations

## E2. Create the trusted SMB source list

sudo tee /var/ossec/etc/lists/ocblue_trusted_smb_sources > /dev/null <<'EOF' 192.168.56.102:approved-hospital-system EOF sudo chown wazuh:wazuh /var/ossec/etc/lists/ocblue_trusted_smb_sources sudo chmod 660 /var/ossec/etc/lists/ocblue_trusted_smb_sources

## E3. Register both lists in ossec.conf

Back up the manager configuration:

sudo cp /var/ossec/etc/ossec.conf /var/ossec/etc/ossec.conf.before-ocblue-lists

Inside the existing <ruleset> section of /var/ossec/etc/ossec.conf, add:

<list>etc/lists/ocblue_suspicious_destinations</list> <list>etc/lists/ocblue_trusted_smb_sources</list>

Keep the existing default lists and place these entries before the User-defined ruleset lines.

# Part F: Install the Current Five-Rule File

## F1. Back up any existing custom rules

sudo cp /var/ossec/etc/rules/operation_code_blue_rules.xml \ /var/ossec/etc/rules/operation_code_blue_rules.xml.backup-$(date +%Y%m%d-%H%M%S)

## F2. Write the complete current rule file

Paste note: Paste the block exactly. EOF must be alone on the final line.

sudo tee /var/ossec/etc/rules/operation_code_blue_rules.xml > /dev/null <<'EOF' <group name="operation_code_blue,capstone,windows,"> <!-- Threat-listed outbound connection to the simulated portal --> <rule id="100099" level="9"> <if_sid>60103</if_sid> <field name="win.system.eventID">^5156$</field> <field name="win.eventdata.destPort">^5000$</field> <list field="win.eventdata.destAddress" lookup="address_match_key">etc/lists/ocblue_suspicious_destinations</list> <description>Operation Code Blue: Hospital workstation connected to threat-listed destination $(win.eventdata.destAddress):$(win.eventdata.destPort)</description> <group>phishing,network_connection,threat_intelligence,suspicious_destination,soc_queue,</group> </rule> <!-- Baseline evidence for protected patient-record activity --> <rule id="100100" level="3"> <if_sid>60103</if_sid> <field name="win.system.eventID">^4663$</field> <field name="win.eventdata.objectName" type="pcre2">(?i)HospitalLab.*PatientRecords</field> <description>Operation Code Blue: Patient record activity by $(win.eventdata.subjectUserName), file: $(win.eventdata.objectName), process: $(win.eventdata.processName)</description> <group>patient_records,file_access,investigation_context,</group> </rule> <!-- Unexpected remote SMB access to PatientRecords --> <rule id="100101" level="9"> <if_sid>60103</if_sid> <field name="win.system.eventID">^5145$</field> <field name="win.eventdata.shareName" type="pcre2">(?i)PatientRecords</field> <list field="win.eventdata.ipAddress" lookup="not_address_match_key">etc/lists/ocblue_trusted_smb_sources</list> <description>Operation Code Blue: PatientRecords accessed from unapproved source $(win.eventdata.ipAddress) by $(win.eventdata.subjectUserName), target: $(win.eventdata.relativeTargetName)</description> <group>patient_records,smb_access,remote_access,unexpected_source,soc_queue,</group> </rule> <!-- Quiet correlation seed: blocked TCP probe against the patient server --> <rule id="100102" level="2"> <if_sid>60104</if_sid> <field name="win.system.eventID">^5152$</field> <field name="win.eventdata.destAddress">^192\.168\.56\.102$</field> <field name="win.eventdata.protocol">^6$</field> <options>no_log</options> <description>Operation Code Blue: Blocked TCP probe from $(win.eventdata.sourceAddress) to protected server port $(win.eventdata.destPort)</description> <group>network_probe,firewall,reconnaissance,</group> </rule> <!-- Rapid multi-port scan from one source --> <rule id="100103" level="10" frequency="40" timeframe="5" ignore="60"> <if_matched_sid>100102</if_matched_sid> <same_field>win.eventdata.sourceAddress</same_field> <different_field>win.eventdata.destPort</different_field> <description>Operation Code Blue: Rapid multi-port scan detected against patient server 192.168.56.102 from $(win.eventdata.sourceAddress)</description> <mitre> <id>T1046</id> </mitre> <group>port_scan,network_service_discovery,reconnaissance,soc_queue,</group> </rule> </group> EOF

# Part G: Validate and Restart Wazuh

sudo /var/ossec/bin/wazuh-analysisd -t sudo systemctl restart wazuh-manager sudo systemctl is-active wazuh-manager

No output from wazuh-analysisd -t means validation passed. The final command should return active.

## G1. Confirm the rules and lists

sudo grep -n -E '100099¦100100¦100101¦100102¦100103' /var/ossec/etc/rules/operation_code_blue_rules.xml sudo cat /var/ossec/etc/lists/ocblue_suspicious_destinations sudo cat /var/ossec/etc/lists/ocblue_trusted_smb_sources

# Part H: Test Every Detection

## H1. Rule 100099 - threat-listed portal connection

KALI VM:

cd ~/operation-code-blue-portal source .venv/bin/activate python3 app.py

WINDOWS VM: open the portal in Edge:

Start-Process msedge "http://identity.blueridgeregional.net:5000/?ioc-test=$(Get-Date -Format yyyyMMddHHmmss)"

Wazuh search:

rule.id:100099

Expected: level 9, threat-listed destination 192.168.56.101:5000, and soc_queue in rule groups.

## H2. Rule 100100 - protected-file investigation context

WINDOWS VM:

$stamp = Get-Date -Format "yyyyMMdd-HHmmss" $file = "C:\HospitalLab\PatientRecords\Group-Test-$stamp.txt" "Fictional patient record" ¦ Set-Content $file Get-Content $file

Wazuh search:

rule.id:100100

Expected: level 3 with the account and protected path. It should not appear in rule.groups:soc_queue.

## H3. Rule 100101 - unapproved SMB source

KALI VM:

smbclient //192.168.56.102/PatientRecords -U ncarter -W DESKTOP-M7O8MTR

At the smb prompt:

ls get "Patient-Access-Test.txt" "/tmp/group-test.txt" quit

Wazuh search:

rule.id:100101

Expected: level 9, source 192.168.56.101, user ncarter, requested target, and soc_queue.

## H4. Confirm an approved source is excluded

WINDOWS VM:

net use \\192.168.56.102\PatientRecords /delete /y net use \\192.168.56.102\PatientRecords /user:DESKTOP-M7O8MTR\ncarter * Get-ChildItem \\192.168.56.102\PatientRecords net use \\192.168.56.102\PatientRecords /delete /y

Refresh rule.id:100101. No new alert should appear for the approved source 192.168.56.102.

## H5. Rules 100102 and 100103 - behavior-based scan

KALI VM:

sudo nmap -sS -Pn -T4 -p 1-200 192.168.56.102

Rule 100102: Quiet level-2 seed. Do not expect it in normal Threat Hunting because no_log is enabled.

Rule 100103: Visible level-10 alert after 40 probes in five seconds from the same source across different ports.

Duplicate suppression: The rule suppresses repeated scan alerts for 60 seconds.

Wazuh search:

rule.id:100103

# Part I: Configure the Analyst SOC Queue

In Threat Hunting > Events, enter:

rule.groups:soc_queue

Set the time range to Last 15 minutes.

Sort newest first for live monitoring or oldest first when presenting the attack timeline.

Save the search as Operation Code Blue - SOC Queue.

Use automatic refresh when available, or click Refresh during the demonstration.

| Visible rule | What the analyst sees |
| --- | --- |
| 100099 | Threat-listed portal connection |
| 100101 | PatientRecords access from an unapproved SMB source |
| 100103 | Rapid multi-port scan against the patient server |

Signal versus context: Rule 100100 remains searchable as supporting file-level evidence but is intentionally excluded from the high-priority SOC queue.

# Part J: Safe Ransomware-Style Rename

This step changes only the filename. It does not encrypt, corrupt, or delete the file.

KALI VM:

smbclient //192.168.56.102/PatientRecords -U ncarter -W DESKTOP-M7O8MTR

At the smb prompt:

rename "Patient-Access-Test.txt" "Patient-Access-Test.txt.locked" ls quit

Restore after the demonstration:

smbclient //192.168.56.102/PatientRecords -U ncarter -W DESKTOP-M7O8MTR \ -c 'rename "Patient-Access-Test.txt.locked" "Patient-Access-Test.txt"'

# Part K: Full Incident Timeline and Troubleshooting

## K1. Combined timeline

rule.id:100103 OR rule.id:100099 OR rule.id:100101 OR rule.id:100100

Use Last 15 minutes and sort oldest to newest. Rule 100100 supplies the file context while 100099, 100101, and 100103 are the analyst-priority signals.

## K2. Common checks

No portal alert: Confirm the portal is running, Windows connects to port 5000, the IOC list contains the destination IP, and Wazuh was restarted after list changes.

No SMB alert: Confirm Event 5145 is forwarded, Kali is not in the trusted list, and the share name contains PatientRecords.

Approved source still alerts: Open the event and add the exact source address Windows reports to the trusted SMB list.

No file context: Verify File System auditing and the PatientRecords folder SACL.

No scan alert: Confirm Event 5152 is forwarded and run the -T4 scan across ports 1-200.

Manager will not start: Run sudo /var/ossec/bin/wazuh-analysisd -t and correct the reported XML or list error before restarting.

# Current Rule Quick Reference

| Rule | Level | Search | Expected purpose |
| --- | --- | --- | --- |
| 100099 | 9 | rule.id:100099 | Threat-listed portal connection |
| 100100 | 3 | rule.id:100100 | Protected-file investigation context |
| 100101 | 9 | rule.id:100101 | Unapproved SMB source |
| 100102 | 2 | Hidden by no_log | Port-scan correlation seed |
| 100103 | 10 | rule.id:100103 | Rapid multi-port scan |
| SOC queue | - | rule.groups:soc_queue | 100099 + 100101 + 100103 |